HNA02-PHL

2MP Covert Network Camera

HNA02-PHL is the covert camera with 4 optional lens shapes.

Key features including:

Cylindrical, block-shaped, fixed focal and ball-shaped of lens are optional;

2m or 8m (between lens and camera body) cable is optional for flexible installation;

HNA02-PHL 1080p HD resolution for real-time output; Rotate mode, BLC and True WDR are supported for different environment.

Key Features:

- 1/2.7” Progressive Scan CMOS

- Up to 1920×1080@30fps for real-time output

- True WDR, up to 120dB

- PoE supported

- Built-in Micro SD / SDHC / SDXC card slot, up to 128GB

- Support HLC, Defog

- Optional appearance of the front-end(the mini bullet and block) and brackets

|  |  |  | Specification |
| --- | --- | --- | --- |
|  | Camera |  |  |
| Image Sensor |  |  | 1/2.7’’ Progressive Scan CMOS |
| Min. Illumination |  |  | 0.01Lux @(F1.2, AGC ON); 0.028 Lux @(F2.0, AGC ON) |
| Shutter Speed |  |  | 1s to 1/100,000s |
| Slow Shutter |  |  | Support |
|  |  |  | L10/L20/10/20: 3.7mm @F2.5, angle of view: 78.5° |
|  |  | L30/30: 2.8mm@F2.0, angle of view: 109.4°/ 4mm @F2.2, angle of view: 90°/ 6 mm @F2.0, |  |
| Lens |  |  | angle of view: 53.9° (Ingress Protection: IP66) |
|  |  | L40: 2.8mm@F2.0, angle of view: 109.4°/ 4mm @F2.2, angle of view: 90°/ 6 mm @F2.0, angle |  |
|  |  |  | of view: 53.9°/ Fisheye: 1.39mm@F2.0, angle of view: 180° (Ingress Protection: IP66) |
| Lens Mount |  |  | M12 |
| WDR |  |  | 120dB |
| DNR |  |  | 3D Digital Noise Reduction |
| Length of Lens Cable |  |  | Optional 2m or 8m |
|  | Compression Standard |  |  |
| Video Compression |  |  | H.264/MJPEG |
| H.264 Type |  |  | Main Profile / High Profile |
| Video Bit Rate |  |  | 32 Kbps to 16 Mbps |
| Audio Compression |  |  | G.711/G.722.1/G.726/MP2L2/PCM |
| Audio Bit Rate |  |  | 64Kbps (G.711)/16Kbps (G.722.1)/16Kbps (G.726)/32 to 160Kbps (MP2L2) |
|  | Image |  |  |
| Max. Resolution |  | 1920×1080 |  |
| Frame Rate |  |  | 50Hz: 25fps (1920×1080, 1280×960, 1280×720) |
|  |  |  | 60Hz: 30fps (1920×1080, 1280×960, 1280×720) |
| Third stream |  |  | Independent with main stream, up to 50Hz: 10fps (1280×960); 60Hz: 10fps (1280×960) |
| Image Setting |  |  | Rotate mode(first channel), saturation, brightness, contrast, sharpness and gray scale |
|  |  |  | adjustable |
| BLC |  |  | Yes, zone optional |
| Defog |  |  | Support |
| Highlight Compensation |  |  | Support |
| Day / Night mode |  |  | Auto / Timing / Alarm trigger |
| Image Overlay |  |  | Support 128×128 size BMP with 24-bit depth image overlay, zone optional |
| ROI |  |  | Up to 4 fixed areas in each stream |
|  | Network |  |  |
| Network Storage |  |  | Support Micro SD/SDHC/SDXC slot, up to 128GB,Edge recording, NAS (NFS,SMB/CIFS) |
|  |  |  | motion detection, face detection, dynamic analysis, tampering alarm, network disconnect, IP |
| Alarm Trigger |  |  | address conflicts, storage full, storage error, Intrusion detection, Line crossing detection, |
|  |  |  | defocus detection, audio exception detection, scene change detection |
| Protocols |  |  | TCP/IP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, PPPoE, NTP, UPnP, SMTP, |
|  |  |  | SNMP, IGMP, 802.1X, QoS, IPv6, Bonjour |
| Standard |  |  | ONVIF(PROFILE S, PEOFILE G), ISAPI |
| General |  |  | Reset, flickerless, multi-stream, heartbeat, rotate, password protection, video masking, |
|  |  |  | watermark, anonymous login, IP address filtering |
|  | Interface |  |  |
| Communication |  |  | 1 RJ45 10M/100M Ethernet interface |
| Audio Interface |  |  | 1 3.5mm audio in (Mic in/Line in)/out interface |
| Alarm Input & Output |  |  | 1-ch alarm input and 1-ch alarm output |

General

| Working Environment | Temperature :-30 °C to 60 °C (-22 °F to 140 °F) |
| --- | --- |
|  | Humidity 95% or less (non-condensing) |
| Power Supply | 12 VDC ± 10%/PoE (802.3af) (Class 3) |
| Power Consumption | Max. 6.5 W |
| Dimensions | 78×105×32 mm (3.07”×4.13”×1.26”) |
| Shipping Weight | 2m cable: <570g |
|  | 8m cable: <680g |
|  | Main body: 210g |
|  | Cylindrical lens: 113g (2m), 233g (8m) |
| Net Weight | Block-shaped lens: 99g (2m), 220g (8m) |
|  | Fixed focal lens: 129g (2m), 242g (8m) |
|  | Ball-shaped lens: 120g (2m), 268.4g (8m) |
|  | Package + accessories: 224g |
|  | Different accessories included for each sensor unit: |
|  | HNA02-PHL |
| Order Model |  |

Dimension

Turret Bracket

In-ceiling Bracket