AC318D-OD4Z

8 MP Dome Camera

Key Features - 8.29 MP high performance CMOS - Auto focus - 3840 × 2160 resolution - 2.7 mm to 13.5 mm motorized vari-focal lens - EXIR 2.0, smart IR, up to 60 m IR distance - OSD menu, DNR, - 4 in 1 video output (switchable TVI/AHD/CVI/CVBS) - IP67 - Up the coax (UTC-C)

| Camera |  |
| --- | --- |
| Image Sensor | 8.29 megapixel progressive scan CMOS |
| Frame Rate | TVI: 8MP@12.5fps, 8MP@15fps, 5MP@20fps, 4MP@25fps, 4MP@30fps, 2MP@25fps, 2MP@30fps CVI: 8MP@12.5fps, 8MP@15fps AHD: 5MP@20fps CVBS: 960H P/N |
| Resolution | 3840 (H) × 2160 (V) |
| Min. illumination | Color: 0.01 Lux @ (F1.2, AGC ON), 0 Lux with IR |
| Shutter Time | PAL: 1/12.5 s to 1/50, 000 s NTSC: 1/15 s to 1/50, 000 s |
| Lens | 2.7 mm to 13.5 mm motorized vari-focal lens |
| Horizontal Field of View | 108.1° to 45.6° |
| Lens Mount | Φ14 |
| Day & Night | IR cut filter |
| Angle Adjustment | Pan: 0° to 355°; Tilt: 0° to 75°; Rotate: 0° to 355° |
| Synchronization | Internal synchronization |
| DWDR (Digital Wide Dynamic Range) | 105 dB |
| Menu |  |
| AGC | Yes |
| D/N Mode | Auto/Color/BW (Black and White) |
| White Balance | Auto/Manual |
| BLC | Yes |
| DWDR (Digital Wide Dynamic Range) | Yes |
| Functions | Brightness, Sharpness, DNR, Mirror, Smart IR |
| Interface |  |
| Video Output | 1 HD analog output |
| Switch Button | TVI/AHD/CVI/CVBS |
| General |  |
| Operating Conditions | -40 °C to 60 °C (-40 °F to 140 °F), humidity: 90% or less (non-condensation) |
| Power Supply | 9 VDC to 15 VDC -A: 9.6 VDC to 14.4 VDC; 19.2 VAC to 28.8 VAC |
| Power Consumption | Max. 6.4 W -A: Max. 8.2W |
| Protection Level | IP67 |
| Material | Metal |
| IR Range | Up to 60 m |
| Communication | Up the coax, Protocol: UTC-C (TVI output) |
| Dimensions | Ø 145.2 mm × 124.1 mm (Ø 7.1" × 4.9") |
| Weight | Approx. 640 g (1.41lb.) |

Dimension

| Accessory |  |  |  |
| --- | --- | --- | --- |
| 1273ZJ-140 Wall Mount | 1275ZJ-SUS Vertical Pole Mount | 1276ZJ-SUS Corner Mount | 1253ZJ-L Rain Shade |
| 1273ZJ-140B Wall Mount | 1241ZJ In-ceiling Mount | 1240ZJ Inclined Ceiling Mount |  |

0100001080605