AC348-FB4

4K Ultra-Low Light Bullet Camera

Key Features - 8.29 MP high performance CMOS - Ultra-low light - 3840 × 2160 resolution - 2.8 mm, 3.6 mm, 6 mm fixed focal lens - 120 dB true WDR, 3D DNR - EXIR 2.0, smart IR, up to 60 m IR distance - IP67 - Up the coax (UTC-C)

| Camera |  |
| --- | --- |
| Image Sensor | 8.29 megapixel progressive scan CMOS |
| Signal System | PAL/NTSC |
| Frame Rate | PAL: 8 MP@12.5fps, 5 MP@20fps, 4 MP@25fps, 1080p@25fps NTSC: 8 MP@15fps, 5 MP@20fps, 4 MP@30fps, 1080p@30fps |
| Resolution | 3840 (H) × 2160 (V) |
| Min. illumination | Color: 0.003 Lux@(F1.2, AGC ON), 0 Lux with IR |
| Shutter Time | PAL: 1/12.5 s to 1/10,000 s NTSC: 1/15 s to 1/10,000 s |
| Slow Shutter | Max. 16 times |
| Lens | 2.8 mm, 3.6 mm, 6 mm fixed focal lens |
| Horizontal Field of View | 102.2° (2.8 mm), 79° (3.6 mm), 50.8° (6 mm) |
| Lens Mount | M12 |
| Day & Night | IR cut filter |
| Angle Adjustment (Bracket) | Pan: 0° to 360°, Tilt:0° to 180°, Rotate: 0° to 360° |
| Synchronization | Internal synchronization |
| WDR (Wide Dynamic Range) | 120 dB |
| Menu |  |
| AGC | Support |
| Day/Night Mode | Auto/Color/BW (Black and White) |
| White Balance | Auto/Manual/Natural |
| Privacy Mask | ON/OFF, 4 programmable privacy masks |
| BLC (Backlight Compensation) | Support |
| 3D DNR (Digital Noise Reduction) | Level 1 to 10 |
| Language | English |
| Functions | Contrast, Sharpness, Saturation, Mirror, Defog, HLC |
| Interface |  |
| TVI Video Output | 1 HD analog output |
| CVBS Video Output | 1 CVBS output (75Ω/BNC) |
| General |  |
| Operating Conditions | -40 °C to 60 °C (-40 °F to 140 °F), humidity: 90% or less (non-condensation) |
| Power Supply | 12 VDC ±25% |
| Power Consumption | Max. 4.8 W |
| Protection Level | IP67 |
| Material | Metal |
| IR Range | Up to 60 m |
| Communication | Up the coax, Protocol: UTC-C (TVI output) |
| Dimensions | Ø 70 mm × 224.7 mm (Ø 2.75" × 8.85" ) |
| Weight | Approx. 630 g (1.4 lb.) |

Specification

| Order Models |  |  |  |
| --- | --- | --- | --- |
| AC348-FB4 |  |  |  |
| Dimension |  |  |  |
| Accessory |  |  |  |
| 1280ZJ-XS Junction Box | 1H18 Video Balun |  |  |